COAL PROJECT REPORT

ATmega328P Reaction Time Game

AVR Assembly Language (ATmega328P)

Computer Organization and Assembly Language

1.  Introduction and Project Overview

This report documents the complete implementation of a Reaction Time Game written in AVR assembly language for the ATmega328P microcontroller. The program measures how quickly a player can press a button after an LED and buzzer signal activate, then rates their performance as Excellent, Good, or Slow over a UART serial connection to a host computer.

The project demonstrates the following core concepts in computer organisation and assembly language:

- Hardware register manipulation for GPIO, UART, and Timer peripherals

- 16-bit arithmetic on an 8-bit processor using register pairs

- Software-defined delay loops calibrated to the 16 MHz system clock

- Stack-based register preservation using PUSH and POP

- Flash memory string storage and retrieval using the Z-pointer and LPM

- Pseudo-random number generation derived from a free-running timer

2.  Hardware Configuration and Pin Mapping

The following table lists every hardware signal used by the program and how it is configured in the initialisation code.

| Pin / Signal | AVR Name | Direction | Purpose |
| --- | --- | --- | --- |
| Arduino D8 | PB0 | Output | LED — visual reaction cue |
| Arduino D9 | PB1 | Output | Buzzer — audio reaction cue (OC1A) |
| Arduino D2 | PD2 | Input | Push-button (active HIGH, external pull-down) |
| Arduino D1 | TX / PD1 | Output | UART transmit to host PC (9600 baud) |
| Timer0 | TCNT0 | Internal | Free-running 8-bit counter for pseudo-random seed |
| Timer1 | OC1A/PB1 | Output | CTC PWM tone generator for buzzer frequencies |

3.  Register Definitions

At the top of the source file, three general-purpose registers are given descriptive aliases using the .def directive. This improves readability without any performance cost because the assembler substitutes the physical register number at assembly time.

| Alias | Physical Register | Role in This Program |
| --- | --- | --- |
| temp | r16 | Primary scratch register — used for loading constants and writing to I/O registers |
| temp2 | r17 | Secondary scratch register — used during 16-bit comparison operations |
| cmp | r22 | UART polling register — temporarily holds the UCSR0A hardware status byte |

Register pairs r24:r25 and r1:r0 appear throughout the code without aliases because the AVR instruction set mandates specific pairs for certain operations (e.g., MUL always writes to r1:r0, and ADIW/SBIW operate on r24–r31 pairs only).

4.  Initialisation Sequence

On power-on, the hardware's reset vector forces execution to address 0x0000. The instruction placed there is:

.org 0x0000

rjmp init

This unconditionally redirects the program counter to the init label, bypassing the interrupt vector table and entering the setup routine. The init routine configures five independent hardware subsystems in order.

4.1  Stack Pointer Initialisation

The stack is a region of SRAM used by RCALL, PUSH, and POP instructions to save and restore data. Before any subroutine can be called, the Stack Pointer (SP) must be set to the top of available SRAM so it has room to grow downward.

ldi temp, HIGH(RAMEND)   ; Load upper byte of last SRAM address

out SPH, temp            ; Write to Stack Pointer High register

ldi temp, LOW(RAMEND)    ; Load lower byte

out SPL, temp            ; Write to Stack Pointer Low register

RAMEND is a constant defined in m328pdef.inc. For the ATmega328P it equals 0x08FF, so after these four instructions the SP points to address 0x08FF. Each PUSH decrements SP by one; each POP increments it.

| Instruction | Operands | Operation Performed |
| --- | --- | --- |
| LDI | reg, k | Load an 8-bit constant k directly into a working register (r16–r31 only) |
| OUT | A, reg | Copy the value of a register to an I/O-space register (address 0x00–0x3F) |
| HIGH | (expr) | Assembler macro that extracts the upper byte of a 16-bit expression |
| LOW | (expr) | Assembler macro that extracts the lower byte of a 16-bit expression |

4.2  GPIO Direction and Initial State

Every I/O pin on the ATmega328P is controlled by three registers per port: DDRx sets direction (1 = output, 0 = input), PORTx drives outputs or enables the internal pull-up on inputs, and PINx reads the live voltage on the pin.

ldi temp, (1<<PB0)|(1<<PB1)   ; Build bitmask: bit 0 and bit 1 set to 1

out DDRB, temp                ; PB0 and PB1 become outputs

cbi PORTB, PB0                ; Drive PB0 low (LED off)

cbi PORTB, PB1                ; Drive PB1 low (buzzer off)

cbi DDRD, PD2                 ; PD2 direction = input

cbi PORTD, PD2               ; Disable internal pull-up (external pull-down used)

| Instruction | Operands | Operation Performed |
| --- | --- | --- |
| LDI | reg, (1<<n)¦(1<<m) | Build a bitmask by ORing shifted 1s — bit n and bit m become 1, all others 0 |
| CBI | A, b | Clear Bit b in I/O register A to 0 (single-cycle atomic instruction for addresses 0x00–0x1F) |
| SBI | A, b | Set Bit b in I/O register A to 1 (complement to CBI) |
| DDRB | — | Data Direction Register for Port B — each bit controls one pin's direction |
| PORTB | — | Port B output latch — writing 1 drives pin HIGH, writing 0 drives pin LOW |

4.3  UART Initialisation (9600 Baud, 8N1)

UART (Universal Asynchronous Receiver-Transmitter) is the hardware module responsible for serial communication. To use it, four registers must be configured: baud rate, transmitter enable, and frame format.

Baud Rate Register (UBRR0)

The UART hardware divides the CPU clock by (UBRR0 + 1) × 16 to produce the baud clock. The correct value is calculated from the manufacturer's formula:

UBRR0 = (F_CPU / (16 × Baud)) - 1

= (16,000,000 / (16 × 9600)) - 1

= 104 - 1 = 103

ldi temp, 0                   ; High byte is 0 (103 fits in one byte)

sts UBRR0H, temp

ldi temp, 103

sts UBRR0L, temp

STS (Store Direct to SRAM) is used here instead of OUT because UBRR0H and UBRR0L reside in extended I/O space (addresses above 0x3F), which is outside the range accessible by the OUT instruction.

Transmitter Enable (UCSR0B)

ldi temp, (1<<TXEN0)

sts UCSR0B, temp              ; Activate the TX hardware block

TXEN0 is bit 3 of UCSR0B. Setting it powers up the transmitter hardware and connects the TX output to pin PD1 (Arduino Digital Pin 1). The bitmask 1<<TXEN0 evaluates to 0b00001000.

Frame Format (UCSR0C)

ldi temp, (1<<UCSZ01)|(1<<UCSZ00)

sts UCSR0C, temp              ; 8-bit character size, 1 stop bit, no parity

The UCSZ bits select the number of data bits per frame. The table below shows all combinations:

| UCSZ02 | UCSZ01 | UCSZ00 | Character Size |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 5-bit |
| 0 | 0 | 1 | 6-bit |
| 0 | 1 | 0 | 7-bit |
| 0 | 1 | 1 | 8-bit ← selected |

| Instruction | Operands | Operation Performed |
| --- | --- | --- |
| STS | k, reg | Store value of reg to any SRAM or extended I/O address k (16-bit address range) |
| LDS | reg, k | Load value from SRAM/extended I/O address k into reg |
| OUT | A, reg | Write to classic I/O space only (addresses 0x00–0x3F); faster than STS but limited range |

4.4  Timer0 Prescaler Configuration

Timer0 is an 8-bit counter that increments continuously once configured. Here it is used solely as an entropy source for the random delay; it is not connected to any output pin in this setup.

ldi temp, (1<<CS01)|(1<<CS00)  ; Prescaler = 64

out TCCR0B, temp

TCCR0B is in classic I/O space so OUT is used. The Clock Select bits CS02:CS01:CS00 select the prescaler ratio:

| CS02 | CS01 | CS00 | Timer0 Clock Source |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Timer stopped |
| 0 | 0 | 1 | No prescaling (16 MHz) |
| 0 | 1 | 0 | CLK / 8 (2 MHz) |
| 0 | 1 | 1 | CLK / 64 (250 kHz) ← selected |
| 1 | 0 | 0 | CLK / 256 (62.5 kHz) |
| 1 | 0 | 1 | CLK / 1024 (15.625 kHz) |

With a prescaler of 64, Timer0 increments at 250,000 counts per second and overflows from 255 back to 0 approximately 977 times per second. The value of TCNT0 at any instant is effectively unpredictable, making it suitable as a random seed.

5.  Main Game Loop

After initialisation, execution falls into the loop: label. Every pass through this label represents one complete round of the game. The loop never terminates — the game restarts indefinitely.

5.1  Pre-round Reset and "Get Ready" Signal

cbi PORTB, PB0             ; Ensure LED is off

rcall tone_stop            ; Ensure buzzer is silent

; Load Z with address of "Get ready..." string

ldi ZL, LOW(str_getready<<1)

ldi ZH, HIGH(str_getready<<1)

rcall print_str            ; Transmit string over UART

ldi r24, LOW(124)          ; Play 1000 Hz tone (OCR = 124)

ldi r25, HIGH(124)

rcall play_tone

rcall delay_200ms          ; Brief audible cue

rcall tone_stop

rcall delay_500ms          ; Short pause before random delay begins

| Instruction | Operation Performed |
| --- | --- |
| RCALL label | Push return address onto stack, then jump to label |
| LDI ZL, LOW(label<<1) | Load the low byte of the byte-address of a Flash label into ZL (r30) |
| LDI ZH, HIGH(label<<1) | Load the high byte into ZH (r31) — ZH:ZL together form the 16-bit Z pointer |

The <<1 (left-shift by 1) is necessary because Flash is word-addressed (each address holds 2 bytes), but strings must be read byte-by-byte. Multiplying the word address by 2 converts it to a byte address.

5.2  Pseudo-Random Delay Generation

To prevent the player from memorising a fixed countdown, the delay before the LED activates is randomised each round using the current value of the free-running Timer0 counter.

in r16, TCNT0         ; Read current Timer0 count (0–255, unpredictable)

ldi r17, 15

mul r16, r17          ; r1:r0 = TCNT0 × 15  (result 0–3825)

movw r24, r0          ; Copy 16-bit result to r25:r24

ldi r16, LOW(1000)    ; Add 1000 to guarantee minimum 1-second delay

ldi r17, HIGH(1000)

add r24, r16

adc r25, r17          ; r25:r24 now holds delay in milliseconds (1000–4825)

| Step | Register Contents | Meaning |
| --- | --- | --- |
| Read TCNT0 | r16 = 0–255 | Unpredictable snapshot of free-running counter |
| MUL r16, r17 | r1:r0 = 0–3825 | Scale up to a larger range |
| MOVW r24, r0 | r25:r24 = 0–3825 | Move 16-bit result into working pair |
| ADD + ADC + 1000 | r25:r24 = 1000–4825 | Guarantee at least 1 second, up to ~4.8 seconds |

| Instruction | Operation Performed |
| --- | --- |
| IN reg, A | Copy value from classic I/O register A into a working register |
| MUL a, b | Unsigned 8×8 multiply; 16-bit result written to r1 (high) and r0 (low) |
| MOVW dst, src | Copy a register pair (src+1:src) to (dst+1:dst) atomically in one cycle |
| ADD a, b | Add register b to register a; sets Carry flag on overflow |
| ADC a, b | Add register b plus the Carry flag to a; used for the high byte of 16-bit addition |

After computing the delay value, the random delay loop counts it down one millisecond at a time:

random_delay_loop:

rcall delay_1ms       ; Spend exactly 1 ms

sbiw r24, 1           ; Subtract 1 from 16-bit pair r25:r24

brne random_delay_loop; Repeat until result reaches zero

| Instruction | Operation Performed |
| --- | --- |
| SBIW reg, k | Subtract Immediate from Word — subtracts a 6-bit constant from a 16-bit pair (r25:r24, r27:r26, r29:r28, or r31:r30); sets Zero flag when result is 0 |
| BRNE label | Branch if Not Equal — jumps to label if the Zero flag is clear |

5.3  Stimulus Activation

sbi PORTB, PB0          ; LED on

ldi r24, LOW(61)        ; OCR value for 2000 Hz

ldi r25, HIGH(61)

rcall play_tone         ; Buzzer on at 2000 Hz

ldi r24, LOW(0)         ; Reset reaction-time counter to zero

ldi r25, HIGH(0)

The OCR value of 61 produces 2000 Hz because: 16,000,000 ÷ (8 × 2 × (61+1)) = 2000. Setting r25:r24 to zero prepares the counter for the reaction time measurement that immediately follows.

5.4  Reaction Time Measurement

Once the LED and buzzer activate, the program enters a polling loop that counts elapsed milliseconds until the button on PD2 is pressed.

wait_press:

sbic PIND, PD2        ; Skip next line if PD2 is LOW (button not pressed)

rjmp press_done       ; If PD2 is HIGH (pressed), exit loop

rcall delay_1ms       ; Wait 1 ms

adiw r24, 1           ; Increment 16-bit counter by 1

rjmp wait_press       ; Go back and check again

| Instruction | Operation Performed |
| --- | --- |
| SBIC A, b | Skip If Bit in I/O register is Clear — skips the next instruction if bit b of I/O register A is 0 |
| RJMP label | Relative Jump — unconditional branch to label (range ±2 KB) |
| ADIW reg, k | Add Immediate to Word — adds a 6-bit constant to a 16-bit register pair; complement to SBIW |
| PIND | Port D Input register — reads the live voltage levels on Port D pins regardless of direction setting |

The counter in r25:r24 increments by 1 per loop iteration, and each iteration takes exactly 1 ms (enforced by delay_1ms). Therefore, when the button is detected, the 16-bit value in r25:r24 is the reaction time in milliseconds.

6.  Result Processing and Grading

6.1  Preserving the Reaction Time on the Stack

After press_done is reached, the first actions are to turn off the LED and buzzer, then protect the reaction time value before any print subroutines corrupt it.

cbi PORTB, PB0          ; LED off

rcall tone_stop         ; Buzzer off

push r24                ; Save low byte of reaction time

push r25                ; Save high byte of reaction time

; ... printing subroutines (which use r24, r25 internally) ...

pop r25                 ; Restore high byte

pop r24                 ; Restore low byte (note LIFO order)

PUSH and POP follow Last-In-First-Out (LIFO) order. Because r24 was pushed first, r25 must be popped first to restore the pair correctly. If the pop order were reversed, r24 and r25 would contain each other's values.

| Instruction | Operation Performed |
| --- | --- |
| PUSH reg | Decrement SP, then write reg to the memory address SP points to |
| POP reg | Read the byte at the address SP points to into reg, then increment SP |

6.2  16-bit Comparison and Branching

An ATmega328P register is 8 bits wide and can hold at most 255. Because reaction times can exceed 255 ms, the value spans two registers (r25:r24). Comparing a 16-bit value against a threshold therefore requires two instructions in sequence.

; Check if reactionTime < 200

ldi temp,  HIGH(200)      ; temp  = 0

ldi temp2, LOW(200)       ; temp2 = 200

cp  r24, temp2            ; Compare low bytes

cpc r25, temp             ; Compare high bytes, accounting for borrow from cp

brlo excellent            ; Branch if r25:r24 < 200

; Check if reactionTime < 400

ldi temp,  HIGH(400)

ldi temp2, LOW(400)

cp  r24, temp2

cpc r25, temp

brlo good

rjmp slow                 ; Falls through if >= 400

| Instruction | Operation Performed |
| --- | --- |
| CP a, b | Compare registers a and b by computing a − b; sets flags but discards result |
| CPC a, b | Compare with Carry — same as CP but also subtracts the Carry flag; used for the high byte of a 16-bit compare to propagate borrow from the low byte |
| BRLO label | Branch if Lower (unsigned) — jumps if the Carry flag is set after CP/CPC, meaning the left operand was strictly less than the right |
| RJMP label | Relative Jump — unconditional branch, used here as the else-fall-through to slow |

6.3  Performance Rating Summary

| Label | Condition | Tone Frequency | Tone Duration | UART Output |
| --- | --- | --- | --- | --- |
| excellent: | reactionTime < 200 ms | 2000 Hz (OCR=61) | 100 ms | Excellent! |
| good: | 200 ms ≤ time < 400 ms | 1200 Hz (OCR=103) | 200 ms | Good |
| slow: | reactionTime ≥ 400 ms | 400 Hz (OCR=311) | 400 ms | Slow |

Each branch prints its rating string via UART, plays a tone of corresponding pitch and duration to reinforce the result acoustically, then falls through to done_round. The done_round label prints a separator line and waits 2000 ms before restarting the loop.

7.  Tone Generation Subroutines

Audio output is produced by Timer1 operating in CTC (Clear Timer on Compare) mode with the OC1A pin (PB1, Arduino D9) toggling on each compare match. The frequency is set by the OCR1A register, which is passed in r25:r24.

7.1  play_tone

play_tone:

ldi temp, (1<<COM1A0)                   ; Toggle OC1A on compare match

sts TCCR1A, temp

ldi temp, (1<<WGM12)|(1<<CS11)|(1<<CS10); CTC mode, prescaler 64

sts TCCR1B, temp

sts OCR1AH, r25                         ; Load compare-match high byte

sts OCR1AL, r24                         ; Load compare-match low byte

ret

The frequency produced by CTC with a toggle output is:

f = F_CPU / (2 × prescaler × (OCR1A + 1))

= 16,000,000 / (2 × 64 × (OCR1A + 1))

| OCR1A Value | Resulting Frequency | Used For |
| --- | --- | --- |
| 124 | ~1000 Hz | "Get ready" alert tone |
| 61 | ~2000 Hz | Stimulus tone and Excellent rating |
| 103 | ~1200 Hz | Good rating tone |
| 311 | ~400 Hz | Slow rating tone |

7.2  tone_stop

tone_stop:

clr temp

sts TCCR1A, temp    ; Disable OC1A output and CTC mode

sts TCCR1B, temp    ; Stop the Timer1 clock

cbi PORTB, PB1     ; Drive PB1 low to silence any residual output

ret

| Instruction | Operation Performed |
| --- | --- |
| CLR reg | Clear register to zero; equivalent to EOR reg, reg — single-cycle, sets Zero flag |

8.  Software Delay Subroutines

All timing in this program is produced by software loops — no hardware timer interrupts are used. The design stacks a single precise 1 ms primitive to build larger delays without duplicating loop logic.

8.1  delay_1ms — The Fundamental Unit

delay_1ms:

push r24

push r25

ldi r24, LOW(3994)

ldi r25, HIGH(3994)

d1_loop:

sbiw r24, 1           ; 2 cycles

brne d1_loop          ; 2 cycles if taken, 1 cycle if fall-through

pop r25

pop r24

ret

Each iteration of d1_loop consumes exactly 4 CPU cycles (2 for SBIW + 2 for BRNE when the branch is taken). The loop count of 3994 is chosen so that the total cycle count including the overhead of PUSH, POP, LDI, RCALL, and RET adds up to exactly 16,000 cycles, which equals 1 ms at 16 MHz.

3994 iterations × 4 cycles = 15,976 cycles

Overhead (push, pop, ldi, rcall, ret) ≈ 24 cycles

Total ≈ 16,000 cycles = 1 ms at 16 MHz

8.2  Composite Delay Functions

All longer delays are built by calling delay_1ms or delay_100ms in counted loops or chains:

| Subroutine | Implementation | Total Duration |
| --- | --- | --- |
| delay_100ms | Loop calls delay_1ms 100 times | 100 ms |
| delay_200ms | Calls delay_100ms twice | 200 ms |
| delay_400ms | Calls delay_200ms twice | 400 ms |
| delay_500ms | Calls delay_400ms once, then delay_100ms once | 500 ms |
| delay_2000ms | Loop calls delay_100ms 20 times | 2000 ms |

| Instruction | Operation Performed |
| --- | --- |
| DEC reg | Decrement register by 1; sets Zero flag when result reaches 0; does not affect Carry |
| BRNE lbl | Branch if the Zero flag is clear (result was not zero) |
| RET | Pop 2-byte return address from stack into PC; returns execution to caller |

9.  UART String Output Subroutines

Three subroutines handle all text output to the serial monitor. They form a layered pipeline: uart_send handles a single byte, print_str_no_newline iterates over a null-terminated string, and print_str adds the CRLF newline sequence.

9.1  uart_send — Single Byte Transmitter

uart_send:

push cmp

us_wait:

lds  cmp, UCSR0A     ; Read UART status register

sbrs cmp, UDRE0      ; Skip next if UDRE0 (Data Register Empty) is set

rjmp us_wait         ; Not empty yet — loop back and check again

sts  UDR0, temp      ; Empty — write byte from temp into transmit register

pop  cmp

ret

The UDRE0 flag in UCSR0A is set to 1 when the UART transmit buffer is empty and ready to accept a new byte. The CPU polls this flag in a tight loop before writing. Without this wait, consecutive writes would overwrite bytes that have not yet been shifted out onto the TX wire.

| Instruction | Operation Performed |
| --- | --- |
| LDS reg, k | Load from extended I/O or SRAM; used here because UCSR0A is above address 0x3F |
| SBRS reg, b | Skip If Bit in Register is Set — skips the next instruction if bit b of reg is 1 |
| STS k, reg | Store value of reg to SRAM/extended I/O address k; used to write to UDR0 |

The cmp register is saved with PUSH at entry and restored with POP at exit. This ensures that whatever value the calling code had in cmp is undisturbed after uart_send returns.

9.2  print_str_no_newline — String Iterator

print_str_no_newline:

lpm  temp, Z+        ; Load byte at Z into temp, then Z++

tst  temp            ; Check if byte is zero (null terminator)

breq ps_end          ; If zero, string is finished — exit

rcall uart_send      ; Otherwise send the character

rjmp print_str_no_newline

ps_end:

ret

| Instruction | Operation Performed |
| --- | --- |
| LPM reg, Z+ | Load Program Memory with post-increment — reads the byte at the Flash address stored in Z into reg, then increments Z by 1 to advance to the next byte |
| TST reg | Test register — performs a logical AND of reg with itself; sets Zero flag if reg is 0 without modifying the register |
| BREQ label | Branch if Equal (Zero flag set) — jumps to label if the last tested value was zero |

9.3  print_str — String with CRLF Newline

print_str:

rcall print_str_no_newline

ldi  temp, 13        ; ASCII 13 = Carriage Return (CR)

rcall uart_send

ldi  temp, 10        ; ASCII 10 = Line Feed (LF)

rcall uart_send

ret

Sending ASCII 13 (CR) moves the cursor to the start of the current line. Sending ASCII 10 (LF) moves it down one line. Together they form the standard CRLF sequence expected by Windows terminal emulators such as PuTTY.

10.  Decimal Number Printing (print_uint16)

The ATmega328P has no hardware division instruction. To convert a 16-bit binary number (e.g., 245) into the ASCII characters '2', '4', '5' for display on the serial monitor, the code uses a technique called repeated subtraction — dividing by successive powers of ten by counting how many times each can be subtracted before the remainder goes negative.

10.1  print_uint16 — Main Digit Extractor

print_uint16:

clr r18                 ; r18 = leading-zero suppression flag (0 = suppress)

ldi r20, HIGH(10000)    ; Extract ten-thousands digit

ldi r21, LOW(10000)

rcall extract_digit

ldi r20, HIGH(1000)     ; Extract thousands digit

ldi r21, LOW(1000)

rcall extract_digit

ldi r20, HIGH(100)      ; Extract hundreds digit

ldi r21, LOW(100)

rcall extract_digit

ldi r20, HIGH(10)       ; Extract tens digit

ldi r21, LOW(10)

rcall extract_digit

ldi r18, 1              ; Force the ones digit to print even if it is 0

ldi r20, 0

ldi r21, 1

rcall extract_digit

ret

The leading-zero suppression flag r18 starts at 0. While r18 is 0, a digit of zero is silently skipped. The first non-zero digit sets r18 to 1, after which all subsequent zeros (including mid-number zeros like the 0 in 205) are printed normally. The ones digit forces r18 = 1 unconditionally so that the value 0 prints as "0" rather than nothing.

10.2  extract_digit — Repeated Subtraction Worker

extract_digit:

clr r19               ; r19 = digit counter (how many times we subtract)

ed_loop:

cp  r24, r21          ; Compare low bytes of score vs. place value

cpc r25, r20          ; Compare high bytes with carry

brlo ed_done          ; If score < place value, we have our digit

sub r24, r21          ; Subtract low byte of place value from score

sbc r25, r20          ; Subtract high byte with borrow

inc r19               ; Count one more subtraction

rjmp ed_loop

ed_done:

tst  r19              ; Is the digit zero?

brne ed_print         ; Non-zero digit — always print

tst  r18              ; Zero digit — check suppression flag

breq ed_skip          ; Flag is 0 → leading zero, skip it

ed_print:

ldi r18, 1            ; Clear suppression flag for all future digits

ldi temp, '0'         ; ASCII 48

add temp, r19         ; 48 + digit = ASCII character

rcall uart_send       ; Transmit character

ed_skip:

ret

| Instruction | Operation Performed |
| --- | --- |
| SUB a, b | Subtract register b from register a; sets Carry flag if borrow occurs |
| SBC a, b | Subtract with Carry — subtracts register b and the Carry flag from a; propagates borrow from the low-byte subtraction to the high byte |
| INC reg | Increment register by 1; does not affect the Carry flag |
| ADD a, b | Add register b to register a; used here to add the digit value to ASCII '0' |

10.3  Worked Example — Printing 245

| Place Value | Subtractions (r19) | Remainder (r25:r24) | r18 After | Character Sent |
| --- | --- | --- | --- | --- |
| 10000 | 0 | 245 | 0 (leading zero, skip) | — |
| 1000 | 0 | 245 | 0 (leading zero, skip) | — |
| 100 | 2 | 45 | 1 (flag set) | '2' |
| 10 | 4 | 5 | 1 | '4' |
| 1 | 5 | 0 | 1 | '5' |

11.  Flash Memory String Table

All constant strings are stored in Flash program memory at the end of the source file using the .db directive. They are not kept in SRAM to conserve the limited 2 KB of data memory available on the ATmega328P.

| Label | Content | Notes |
| --- | --- | --- |
| str_ready | Reaction Game Ready... | Printed once at startup |
| str_getready | Get ready... | Printed at the start of each round |
| str_reaction | Reaction Time: | Printed without newline before the number |
| str_ms | ms | Printed with newline after the number |
| str_excellent | UTF-8 emoji + ' Excellent!' | 3-byte UTF-8 fire emoji followed by text |
| str_good | UTF-8 emoji + ' Good' | 3-byte UTF-8 emoji followed by text |
| str_slow | UTF-8 emoji + ' Slow' | 3-byte UTF-8 emoji followed by text |
| str_sep | ------------------- | Visual separator between rounds |

Each string is terminated with an explicit 0 byte (null terminator). Strings are padded to an even number of bytes with a second 0 where necessary because Flash words are 16 bits wide and the assembler fills incomplete words with 0.

The 4-byte sequences starting with 0xF0 in str_excellent, str_good, and str_slow are 4-byte UTF-8 encoded emoji characters (Unicode code points above U+FFFF). These display correctly in terminal emulators that support UTF-8 but appear as garbled characters in terminals that do not.

12.  Complete Instruction Reference

The table below consolidates every AVR instruction used in this project with its exact behaviour, flags affected, and cycle count.

| Instruction | Full Name | Operation | Flags |
| --- | --- | --- | --- |
| LDI reg, k | Load Immediate | reg ← k (constant). Only r16–r31. | None |
| OUT A, reg | Output to I/O | I/O[A] ← reg. Addresses 0x00–0x3F only. | None |
| IN reg, A | Input from I/O | reg ← I/O[A]. Addresses 0x00–0x3F only. | None |
| STS k, reg | Store to SRAM/Ext. I/O | SRAM[k] ← reg. Full 16-bit address range. | None |
| LDS reg, k | Load from SRAM/Ext. I/O | reg ← SRAM[k]. Full 16-bit address range. | None |
| SBI A, b | Set Bit in I/O | I/O[A].b ← 1. Addresses 0x00–0x1F only. | None |
| CBI A, b | Clear Bit in I/O | I/O[A].b ← 0. Addresses 0x00–0x1F only. | None |
| SBIC A, b | Skip if Bit in I/O Clear | Skip next instruction if I/O[A].b = 0. | None |
| SBRS reg, b | Skip if Bit in Reg Set | Skip next instruction if reg.b = 1. | None |
| MOV a, b | Copy Register | a ← b. | None |
| MOVW dst, src | Copy Register Word | dst+1:dst ← src+1:src. | None |
| CLR reg | Clear Register | reg ← 0. Equivalent to EOR reg, reg. | Z,N,V |
| TST reg | Test Register | Sets flags for reg AND reg without storing. | Z,N,V |
| INC reg | Increment | reg ← reg + 1. | Z,N,V,S |
| DEC reg | Decrement | reg ← reg − 1. | Z,N,V,S |
| ADD a, b | Add without Carry | a ← a + b. | Z,C,N,V,H |
| ADC a, b | Add with Carry | a ← a + b + C. | Z,C,N,V,H |
| SUB a, b | Subtract without Carry | a ← a − b. | Z,C,N,V,H |
| SBC a, b | Subtract with Carry | a ← a − b − C. | Z,C,N,V,H |
| ADIW pair, k | Add Immediate to Word | pair+1:pair ← pair+1:pair + k. k ≤ 63. | Z,C,N,V,S |
| SBIW pair, k | Subtract Immediate from Word | pair+1:pair ← pair+1:pair − k. k ≤ 63. | Z,C,N,V,S |
| CP a, b | Compare | Sets flags for a − b; result discarded. | Z,C,N,V,H |
| CPC a, b | Compare with Carry | Sets flags for a − b − C; result discarded. | Z,C,N,V,H |
| MUL a, b | Multiply Unsigned | r1:r0 ← a × b (16-bit product). | Z,C |
| BRNE label | Branch if Not Equal | PC ← label if Z = 0. | — |
| BREQ label | Branch if Equal | PC ← label if Z = 1. | — |
| BRLO label | Branch if Lower (unsigned) | PC ← label if C = 1. | — |
| RJMP label | Relative Jump | PC ← PC + offset. Range ±2047 words. | — |
| RCALL label | Relative Call | Push PC+1 onto stack, then PC ← label. | — |
| RET | Return from Subroutine | Pop return address from stack into PC. | — |
| PUSH reg | Push Register | SP ← SP−1; SRAM[SP] ← reg. | None |
| POP reg | Pop Register | reg ← SRAM[SP]; SP ← SP+1. | None |
| LPM reg, Z+ | Load Program Memory (post-inc) | reg ← Flash[Z]; Z ← Z+1. | None |

13.  Compiling and Uploading the Firmware

After assembling the source file to a HEX file, use the following AVRDUDE command to upload it to the Arduino Uno over the USB serial bootloader:

avrdude -C "%AVRDUDE_CONF%" -c arduino -p m328p -P COM4 -b 115200 -U flash:w:"Reaction game.hex":I

| Flag | Value | Meaning |
| --- | --- | --- |
| -C | %AVRDUDE_CONF% | Path to the AVRDUDE configuration file (set by the Arduino IDE environment variable) |
| -c | arduino | Programmer type — uses the STK500v1 bootloader protocol |
| -p | m328p | Target device — ATmega328P |
| -P | COM3 | Serial port the Arduino is connected to (adjust as needed) |
| -b | 115200 | Baud rate for the bootloader handshake |
| -U | flash:w:"...".hex:I | Operation: write (:w) to flash memory from an Intel HEX (:I) file |

14.  Annotated Code Listing

The following table presents every instruction in the program alongside a concise description of what it does at execution time.

| Line / Label | Code | What It Does |
| --- | --- | --- |
| .def definitions | .def temp = r16 | Names r16 as 'temp' throughout the program |
|  | .def temp2 = r17 | Names r17 as 'temp2' |
|  | .def cmp = r22 | Names r22 as 'cmp' |
| Reset vector | .org 0x0000 / rjmp init | Places an unconditional jump at the hardware reset address |
| Stack init | ldi/out SPH, SPL | Loads RAMEND (0x08FF) into the 16-bit stack pointer |
| GPIO setup | ldi temp, (1<<PB0)¦(1<<PB1) / out DDRB, temp | Sets PB0 and PB1 as outputs (LED and buzzer) |
|  | cbi PORTB, PB0 / cbi PORTB, PB1 | Drives both output pins low (off) |
|  | cbi DDRD, PD2 / cbi PORTD, PD2 | Sets PD2 as input, disables internal pull-up |
| UART baud rate | sts UBRR0H, 0 / sts UBRR0L, 103 | Sets baud-rate divisor for 9600 baud at 16 MHz |
| UART enable | ldi temp, (1<<TXEN0) / sts UCSR0B, temp | Enables the UART transmitter hardware |
| UART frame format | ldi temp, (1<<UCSZ01)¦(1<<UCSZ00) / sts UCSR0C, temp | Selects 8-bit data, 1 stop bit, no parity |
| Timer0 prescaler | ldi temp, (1<<CS01)¦(1<<CS00) / out TCCR0B, temp | Starts Timer0 with ÷64 prescaler (entropy source) |
| loop: | cbi PORTB, PB0 / rcall tone_stop | Resets LED and buzzer at the start of each round |
|  | Z load + rcall print_str | Prints 'Get ready...' over UART |
|  | ldi r24/r25, 124 / rcall play_tone | Plays 1000 Hz alert tone |
|  | rcall delay_200ms / tone_stop / delay_500ms | Plays tone for 200 ms then pauses 500 ms |
| Random delay | in r16, TCNT0 | Reads the free-running Timer0 value as a random seed |
|  | ldi r17, 15 / mul r16, r17 / movw r24, r0 | Multiplies seed by 15; result in r25:r24 |
|  | add r24/adc r25 with 1000 | Adds 1000 to guarantee minimum 1-second delay |
| random_delay_loop: | rcall delay_1ms / sbiw r24, 1 / brne | Counts down delay in 1 ms steps |
| Stimulus | sbi PORTB, PB0 | Turns LED on — player must react now |
|  | ldi r24/r25, 61 / rcall play_tone | Starts 2000 Hz stimulus tone |
|  | ldi r24/r25, 0 | Resets counter to zero for reaction timing |
| wait_press: | sbic PIND, PD2 | Checks if button is pressed (PD2 HIGH = pressed) |
|  | rjmp press_done | Exits loop immediately when button is detected |
|  | rcall delay_1ms / adiw r24, 1 / rjmp wait_press | Adds 1 ms to counter each iteration |
| press_done: | cbi PORTB, PB0 / rcall tone_stop | Turns off LED and buzzer |
|  | push r24 / push r25 | Saves reaction time before print subroutines can corrupt it |
|  | Z + print_str_no_newline | Prints 'Reaction Time: ' without newline |
|  | rcall print_uint16 | Converts and prints the 16-bit millisecond value as decimal text |
|  | Z + rcall print_str | Prints ' ms' with newline |
|  | pop r25 / pop r24 | Restores reaction time from stack |
| Grading — cp/cpc/brlo | ldi temp HIGH(200) + cp r24 + cpc r25 + brlo excellent | 16-bit compare: branches to excellent if time < 200 ms |
|  | same for 400 / brlo good / rjmp slow | Branches to good if time < 400 ms, otherwise falls to slow |
| excellent: | print + play 2000 Hz 100 ms | Prints Excellent!, plays high tone |
| good: | print + play 1200 Hz 200 ms | Prints Good, plays mid tone |
| slow: | print + play 400 Hz 400 ms | Prints Slow, plays low tone |
| done_round: | print separator / delay_2000ms / rjmp loop | Prints divider, waits 2 s, loops back |
| play_tone: | TCCR1A/B and OCR1AH/L setup | Configures Timer1 CTC for the frequency in r25:r24 |
| tone_stop: | clr + sts TCCR1A/B + cbi PB1 | Disables Timer1 and drives buzzer pin low |
| delay_1ms: | push/ldi 3994/d1_loop sbiw-brne/pop/ret | Software busy-wait calibrated to exactly 1 ms |
| delay_100ms: | ldi 100 / loop rcall delay_1ms / dec / brne / ret | Calls delay_1ms 100 times |
| delay_200/400/500/2000ms: | chains of smaller delays | Built by composing delay_100ms calls |
| print_uint16: | clr r18 / five extract_digit calls | Extracts and prints each decimal digit 10000–1 |
| extract_digit: | clr r19 / cp-cpc-sub-sbc loop / print logic | Counts repeated subtractions to find one digit; handles leading-zero suppression |
| print_str: | rcall print_str_no_newline + CR(13) + LF(10) | Prints null-terminated Flash string then CRLF |
| print_str_no_newline: | lpm Z+ / tst / breq / rcall uart_send loop | Iterates over Flash string byte by byte |
| uart_send: | push cmp / poll UDRE0 / sts UDR0 / pop cmp / ret | Waits for UART buffer empty then transmits one byte from temp |